### ADB Europe DS205 Zoom Profile with condenser: 2000W 13-36°

The DS205 luminaire is a variable-angle profile spotlight based on high-efficiency optics, equipped with individually adjustable zoom lenses. It provides a sharp or soft edged beam over a very useful range of 13° to 36°; focusing is smooth and stable in any position.

The use of high quality condenser optics results in a superior beam quality and extremely accurate imaging capabilities. All lenses feature anti-reflection coating.

### Photometric data

|  | ADB Europe Series DS205 (2000W 52,000 lm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Miniumum angle: 13° |  |  |  |  |  |  |  |  |  | Maximum angle: 36° |  |  |  |  |  |  |  |  |
|  | Distance (m) | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 |  | Distance (m) | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 |
|  | Beam Dia (m) | 1.4 | 2.0 | 2.7 | 3.4 | 4.1 | 4.8 | 5.5 | 6.2 |  | Beam Dia (m) | 3.9 | 5.9 | 7.8 | 9.8 | 11.7 | 13.7 | 15.6 | 17.6 |
|  | Illumination (lux) | 18470 | 8210 | 4620 | 2960 | 2050 | 1510 | 1155 | 910 |  | Illumination (lux) | 2920 | 1300 | 730 | 470 | 325 | 240 | 182 | 144 |

### Mechanical Information

|  | Note: HWD | DS205 | Packing |
| --- | --- | --- | --- |
|  | Height (mm) | 500 | 835 |
|  | Width (mm) | 420 | 450 |
|  | Depth (mm) | 810 | 370 |
|  | Weight (kg) | 22.9 | 27.2 |

### Supplied with

- 1.5 m silicone power supply cable (2x 1.5 + 1.5 mm²) with CEE 22 connector, without plug

- 1 integrated wire mesh safety guard

- 1 metal filter frame (245 x 245 mm - DIN 15560)

### Optical System

- Sherical reflector of pure, polished, anodised aluminium

- Aspeherical condenser lens

- Intermediary spherical lens

- 13-36° zoom optics consisting of 3 polished plano-convex lenses

- All lenses with anti-reflection coating

- Optional zoom glass diffuser for sharp to soft beam edge conversion

- GY16 lampholder for 220/240 V 2000 W tungsten-halogen lamp with 70 mm light centre length (LCL)

- Focusing mechanism with smooth control via screw knob

### Accessories

- Set of four cardboard filter frames

- Metal filter frame

- Glass diffuser

- Hookclamp

- Slanted yoke

- 1.2 m safety chain

- 1.2 m safety bond

### Options

- Pole operation on pan & tilt

- Pole operation on focus

- Motorisation of luminaire